Firmware Programming Notes for AUTO-OHM/DMOM 200 Series 3

Please use the following steps to program the AUTO-OHM/DMOM S3 firmware.

1. Copy the “AOHMS3_M.HEX” file to the thumb drive.

2. Turn off AUTO-OHM/DMOM S3 power.

3. Insert thumb drive into the AUTO-OHM/DMOM S3 USB thumb drive port on the front panel.

4. Hold “STOP” key on the ATRT-01 S3 key pad and turn on power.

5. When “ERASE” message is shown on the LDC, release “STOP” key.

6. Unit will reprogram itself.

7. Verify new firmware revision when the AUTO-OHM S3 restarts itself.

Contact Vanguard Instruments Company at 909-923-9390 or www.vanguard-instruments.com if you have any questions.