Firmware Programming Notes for DigiTMR S2

Please use the following steps to program the DigiTMR S2 firmware.

1. Copy the “DigiS2_m.HEX” file to the thumb drive.

2. Turn off DigiTMR S2 power.

3. Insert thumb drive into the DigiTMR S2 USB thumb drive port on the front panel.

4. Hold “STOP” key on the DigiTMR S2 key pad and turn on power.

5. When “ERASE” message is shown on the LDC, release “STOP” key.

6. Unit will reprogram itself.

7. Verify new firmware revision when the DigiTMR S2 restarts itself.

Contact Vanguard Instruments Company at 909-923-9390 or www.vanguard-instruments.com if you have any questions.