Firmware Programming Notes for DMOM-600

Please use the following steps to program the DMOM-600 firmware.

1. Copy the “DM600_M.HEX” file to the thumb drive.

2. Turn off DMOM-600 power.

3. Insert thumb drive into the DMOM-600 USB thumb drive port on the front panel.

4. Hold “STOP” key on the DMOM-600 key pad and turn on power.

5. When “ERASE” message is shown on the LDC, release “STOP” key.

6. Unit will reprogram itself.

7. Verify new firmware revision when the DMOM-600 restarts itself.

Contact Vanguard Instruments Company at 909-923-9390 or www.vanguard-instruments.com if you have any questions.