Firmware Programming Notes for TRM-20/40

Please use the following steps to program the TRM firmware.

1. Copy the “TRM_M.HEX” file to the thumb drive.

2. Turn off TRM power.

3. Insert thumb drive into the TRM USB thumb drive port on the front panel.

4. Hold “STOP” key on the TRM key pad and turn on power.

5. When “ERASE” message is shown on the LDC, release “STOP” key.

6. Unit will reprogram itself.

7. Verify new firmware revision when the TRM restarts itself.

Contact Vanguard Instruments Company at 909-923-9390 or www.vanguard-instruments.com if you have any questions.